

TB20GT N113AC CHECKLIST			       Edition 11

23/FEB/2005

General

IN WINDY CONDITIONS, hold door firmly all the way up during opening or closing.

Doors must be closed immediately upon entering or leaving the aircraft, for same reason.

On taxiways, check all grass surfaces and especially grass/concrete transitions BEFORE TAXI.

Do not clean windows with anything other than clean unused cloth and the correct liquid cleaner.

NEVER use paper tissues on windows.

Do not place headsets on top of instrument panel as they scratch the window.

Push/pull the aircraft with the towbar and not by the propeller.

Adjust seats gently - do not ram them against the stops while sliding them.

NEVER turn the propeller backwards (damages the vacuum pump).

Pitot heater gets very hot, very quickly!!!

No loose objects in cockpit - can fall down and jam the rudder pedals.

Keep CHT below 400F at all times if possible.

(OK to drop RPM to say 2500 for less noise); do not lean at any power setting above 75%.

Never assume the parking brakes are ON from the last time you set them

- always re-set them before engine start.

For the official checklist and emergency procedures, see the POH.

Write down Shadin flowmeter FOB reading (USG) before & after flight

Equipment carried

Anywhere near the sea	RAFT, LIFE JACKETS, EPRIB & brief passengers

Over the sea		As above, plus WEAR LIFE JACKETS

Abroad			FILE ICAO FLIGHT PLAN, DOCUMENTS, VAT INVOICE

Speeds (kt)

Vx			81

Vy			95

Vs0 (landing flap)		59

Vs1 (clean)		70

Va			129

Vlo			129

Vle			139

Vfe(10)			129

Vfe(40)			103

Vne			187

Best glide			92 (clean)

Capacities

Usable fuel		86.2 usg (327 litres)

Oil			9 qts

MTOW			3086 lb (1400 kg)

Empty weight		1981lb (900kg)

Useful load (incl. fuel)	1105 lb (500kg)

Fuel weight		517 lb (234kg)

Useful load (with max fuel)	585 lb (266 kg)

External



Front view		Does aircraft sit straight?

Towbar/chocks		IN THE BOOT

Walk around the aircraft and:	Check for any damage

Check everything that sticks out

Check everything that moves

Check all pipes for leakage

Check all hinges

Check vents for obstructions

Fuel (2)			CHECK & NOTE LEVELS & SECURE CAP

Fuel filter drains (3)		FUEL FREE OF WATER, NO LEAKS

Tyres (3)			CHECKED

Gear doors (3)		CHECK PLAY

Gear microswitches	(4)	CLEAN, NO DEBRIS

Gear compartments (3)	CLEAR

Baggage door		LOCKED

Elevator			CHECK LINKAGE & HINGES

CHECK UNDERNEATH & INSIDE FUSELAGE

Rudder			CHECK TRIM LINKAGE & HINGES

Pitot cover		REMOVED

Static vent plugs (2)		REMOVED

Internal & Electrics

Control lock		REMOVED

Fire extinguisher		CHECK PRESSURE (IN GREEN)

Fuel selector		OPEN (LEFT TANK)

Main switch		OFF



Alternator field		OFF

Fuel pump ... Landing light	OFF  (7 switches all in line)

Magneto switch		OFF, KEYS REMOVED

Landing gear switch		DOWN

Main switch		ON

Fuel gauges		Matches actual fuel levels

Shadin flowmeter		FOB matches actual fuel levels & WRITE IT DOWN

Voltage			YELLOW (24V)

Annunciator panel		CHECK FOR WARNINGS

Landing gear lights (3)	3 GREENS

Flaps			LANDING

Strobes ... Landing light	ON  (5 switches in line)

All external lights		CHECK

Stall warning switch		CHECK

Pitot 			HOT

Strobes ... Landing light	OFF (5 switches in line)

Flaps			TEST 2 POSITIONS, RETRACT

Instrument illumination (3)	CHECK & OFF

Quick GPS & VOR Setup

Radio Master		ON

KLN94 GPS		Enter or select flight plan (01..19) & select Use

Check selected flight plan appears under FPL0

and appears correctly on KMD550

COM/NAV & ADF		Select frequencies

Radio Master		OFF

Before engine start

Doors			CLOSED & LOCKED

Cabin heat controls		CLOSED

Seats			ADJUSTED & LOCKED

Seat belts			ADJUSTED & SECURE

Flight controls		CHECK FULL MOVEMENT

Pitch trim			CHECK FULL MOVEMENT

Rudder trim		CHECK FULL MOVEMENT, SET TO TAKEOFF

Fuel selector		OPEN, ON LESS FULL TANK

Emergency gear control	PUSHED IN

Alternate static source	PUSHED IN

Alternate induction air	PUSHED IN

Main switch		ON

# Parking brake		SET, "PARK" LIGHT ON

COLD engine starting

Main switch		ON

Stormscope ... Horizon	OFF (4 switches in line including Autopilot)

Parking brake		SET, "PARK" LIGHT ON

Strobes			ON

Alternator field		OFF

Circuit breakers		CHECK ALL IN

Throttle			1/4 INCH OPEN

Propeller pitch		FULL FORWARD

Mixture			IDLE CUT OFF

Fuel pump		ON



Mixture			FULL RICH until fuel flow displayed (or 7 secs max)

then IDLE CUT OFF

Fuel pump		OFF

Outside			NOBODY NEAR

Ignition switch		BOTH & PRESS TO START (30 secs max)

When engine starts - do immediately:

Ignition switch		RELEASE

Mixture			FULL RICH

Throttle			SET 1200 RPM

Oil pressure		GREEN SECTOR within 30 secs otherwise shut down engine

HOT engine starting

Main switch		ON

Stormscope ... Horizon	OFF (4 switches in line including Autopilot)

Parking brake		SET, "PARK" LIGHT ON

Strobes 			ON

Alternator field		OFF

Circuit breakers		CHECK ALL IN

Throttle			FULL POWER

Propeller pitch		FULL FORWARD

Mixture			IDLE CUT OFF

Fuel pump		ON

Mixture			FULL RICH for 1 sec, then IDLE CUT OFF

Fuel pump		OFF

Outside			NOBODY NEAR/BEHIND

Ignition switch		BOTH & PRESS TO START (30 secs max)

When engine starts - do immediately:

Ignition switch		RELEASE

Mixture			FULL RICH

Throttle			REDUCE TO GET 1200 RPM

Oil pressure		GREEN SECTOR within 30 secs otherwise shut down engine

If engine does not start (after cold or hot start)

Treat as flooded. Crank for a few seconds with throttle MAX, Mixture OFF and ignition OFF.

After engine starting

Alternator field		OFF

ALT warning light		ON

Voltmeter			YELLOW SECTOR

Alternator field 		ON

ALT warning light		OFF

Voltmeter			GREEN SECTOR

Digital battery voltage	27 to 29V

Charging current		POSITIVE READING

Turn coordinator		ON & CHECK FLAG

Stormscope ... Horizon	ON (4 switches in line including Autopilot)

All individual avionics	ON

Vacuum gauge		CHECK (GREEN)

Panel lamp test		CHECK

Landing gear lamp test	CHECK (4 LAMPS)



Fuel selector		OPEN, ON LESS FULL TANK

Avionics setup & test

DAVTRON 		Set to OAT & degC

SHADIN FLOWMETER	Check flow rate matches analog gauge

Check & note FOB, set new FOB if fuel added

Altimeters (2)		Set QFE/QNH as required

HSI & RMI slaving		Check slaving by operating CW/CCW switch

Return switch set to SLAVE

Check both against compass

COM1/2			Set frequencies

NAV1/2			Set to nearest VOR & identify if poss

HSI			Heading bug to runway heading

NAV/GPS switch to NAV

Rotate OBS, check CDI centred on radial to VOR (2 places)

OBS to initial heading

CDI #2			Rotate OBS, check CDI centred on radial to VOR (2 places)

OBS to initial heading

RMI			Set ADF to nearest NDB (ADF button IN), identify if poss

Check both needles point the right way

TRANSPONDER		SBY (standby)

VFR (7000)

DME			Set to NAV2, check reading

Set to NAV1, check reading & identify

INTERCOM		Toggle switches central (2)

Illuminated buttons all OFF except COM1

Test ILS marker lights (T/M)

Rotary knob to COM1

Adjust intercom volume (does P1-P2 only)

GPS			Check satellite reception (AUX1)

Check FPL0 is correct for route

Check FOB (AUX15) is correct

MFD			Check present location & flight plan showing

Test Stormscope

Clear Stormscope

Autopilot			CHECK SELF TEST COMPLETE

Autopilot			ENGAGE WITH "AP" BUTTON

Flight controls		MOVE (to check autopilot clutches can be overpowered)

Autopilot			DISENGAGE (PRESS RED BUTTON) & check audio warning

Autopilot			Set initial cruise altitude

Taxi

ATIS			WRITE DOWN

Clearances		GET & WRITE DOWN

Area			CLEAR TO TAXI

Oil temperature 		GREEN (avoid > 1200RPM if YELLOW)

Taxi light			ON

Brakes			OFF

In turns:			Turn Coordinator shows turn & slip

Horizons (2) remain level

Compass, HSI, RMI, GPS map(s) all rotate

RMI indications (2) remain constant

Engine tests

Parking brakes		SET

Friction			SET

Oil temperature & pressure	GREEN

Mixture			RICH

Fuel			PUMP ON / CHANGE TO FULLEST TANK / PUMP OFF

Propeller			FULL FORWARD

Area			CLEAR BEHIND

Throttle			2000RPM (CHECK BRAKES HOLDING)

Propeller			Cycle twice: All the way back for just long enough to see a

500RPM drop, return immediately to FULL FORWARD

Magnetos:		CHECK L then BOTH

CHECK R then BOTH

Max 175 RPM drop, Max 50 RPM difference

Alternate air		PULL

Manifold pressure		MAINTAINED

Alternate air		PUSHED BACK IN

EDM700			CHECK EGTs LEVEL

Alternator			NO "ALT" LIGHT, POSITIVE CURRENT



Vacuum gauge		CHECK GREEN

Idle check		Slowly reduce to idle, check vacuum & oil pressure hold up

Throttle			1200 RPM

Before takeoff

Seat belts			SECURE

Doors			CLOSED & LOCKED

Controls			CHECK MOVEMENT

Electric trim		CHECK

Pitch trim			TAKEOFF

Rudder trim		TAKEOFF

Flaps			TAKEOFF & CHECK VISUALLY

Magnetos			BOTH

Alternate air		PUSHED

Propeller			FULL FORWARD

Mixture			FULL RICH

Fuel selector		FULLEST TANK

Fuel pump		ON

Oil temperature		GREEN

Oil pressure		GREEN

Voltmeter			GREEN

Altimeter (2)		QFE/QNH SET

HSI & RMI		Check against compass

2nd Horizon		LEVEL

Heater controls		AS REQUIRED

Strobes			ON

Navigation lights		ON

Pitot heat			AS REQUIRED

Landing light		ON

Transponder		7000, ALT

Autopilot			PRESS RED BUTTON & CHECK DISENGAGED

Pitch trim		TAKEOFF

IFR/Takeoff clearance	GET

Takeoff

Lined up on runway		Check heading (compass)

Mixture			FULL RICH

Propeller			FULL FORWARD

Throttle			2000RPM then FULL POWER while keeping it straight

Rotate			70KT

Climb			80KT

When safely airborne & no usable runway left:

Brakes			APPLY

Landing gear		RETRACT

At 300ft:

Flaps			CHECK SPEED > 85KT, THEN RETRACT

At 1000ft:

Fuel pump		OFF

Taxi/Landing lights		OFF

Climb

Optimum climb speed	95KT (105KT gives better engine cooling)

Engine			FULL MP / 2500 RPM / FULL MIXTURE

Do not lean mixture if above 75% power

Set speed to maintain CHT below 400F

Cruise

Rudder trim		AS REQUIRED

Engine/Mixture		See POH Engine Ratings Table - below are for 2000ft



75% power: 24MP / 2400RPM / 14USG  147KT IAS

65% power: 22MP / 2300RPM / 12USG  140KT IAS

55% power: 20MP / 2200RPM / 10.5USG  130KT IAS
			45% power: 21MP / 1800RPM / 8.5USG  120KT IAS

GPS/NAV		ON TRACK

Autopilot			AS REQUIRED (RED button to disengage)

DO NOT USE AP BELOW 1000FT AGL

Flight in turbulence

Speed			130kt (at MTOW)

Seat belts		SECURE

En-route checks:

Fuel selector		Change every 30 mins (with fuel pump ON)

Fuel gauges		SUFFICIENT

GPS			AUX15, AUX17, AUX18 show useful fuel info

Engine			Check EDM700 (CHT < 400F, EGT < 1500F)

Check oil temp & pressure GREEN

Altimeters		CHECK FOR MSA

Alternator			CHARGING (positive current)

Vacuum			CHECK (GREEN)
Outside Temp		CHECK, USE MAX PROP DE-ICE IF BELOW ZERO
Ice			Visual check

Descent

Altimeters		SET QNH

MP			Reduce by MAX 5" per minute

(until @ desired altitude @ 110kt)

If using ILS:
NAV/GPS switch		NAV
NAV1			ILS

NAV2			ILS

DME			NAV1

Approach (non-ILS)

Speed			110 KT (18"MP / 2300RPM / FULL RICH)

Landing light		ON

Brakes			OFF

Landing gear		DOWN
Flaps			TAKEOFF
Throttle			20"MP

Fuel pump		ON

Fuel selector		ON TANK WITH ENOUGH FUEL

Seat belts			SECURE

Doors			CLOSED & LOCKED
Landing gear lights		3 x GREEN

On base leg:

Trim			90KT

Mixture			FULL RICH

Propeller			FULL FORWARD

# FINAL call & check RED / BLUE / 3 GREENS

Flaps			LANDING

Final			75KT

Approach (ILS, non-autopilot)

Speed			110 KT (18"MP / 2300RPM / FULL RICH)

Landing light		ON

Brakes			OFF

Fuel pump		ON

Fuel selector		ON TANK WITH ENOUGH FUEL

Seat belts			SECURE

Doors			CLOSED & LOCKED
At glideslope intercept:

Landing gear		DOWN
Flaps			TAKEOFF
Landing gear lights		3 x GREEN

Power/Trim		Reduce gradually to 90KT

At runway visual:
Flaps			LANDING

Mixture			FULL RICH

Propeller			FULL FORWARD
VISUAL call & check RED / BLUE / 3 GREENS

Final			75KT

Approach (ILS, KFC225 autopilot)

Autopilot			HDG,ALT
Power			110 KT (18"MP / 2300RPM / FULL RICH)

Landing light		ON

Brakes			OFF

Fuel pump		ON

Fuel selector		ON TANK WITH ENOUGH FUEL

Seat belts			SECURE

Doors			CLOSED & LOCKED
At LOC 3/4 deflection	APR (shows APR ARM)
At glideslope intercept:

Landing gear		DOWN
Flaps			TAKEOFF
Landing gear lights		3 x GREEN

Power			Reduce gradually to 90KT

At runway visual:
Autopilot			DISCONNECT WITH RED BUTTON
Flaps			LANDING

Mixture			FULL RICH

Propeller			FULL FORWARD
VISUAL call & check RED / BLUE / 3 GREENS

Final			75KT

Go-around

Throttle			FULL POWER

Pitch			UP (IF NECESSARY)

Landing gear		UP

Flaps			CHECK SPEED > 75KT, THEN TAKEOFF

Airspeed			90KT

Flaps			RETRACT

Climb			95KT

After landing

Past runway threshold	CHECK & STOP

Throttle			1200 rpm

Fuel pump		OFF

Flaps			RETRACT

Pitot heat			OFF

Taxi light			ON

Landing light		OFF

All avionics not required	OFF

Trims (2)			TAKEOFF

Shutdown

Parking brake		ON

Throttle			1200 rpm

Lights			OFF

Shadin flowmeter		Note FOB

Stormscope ... Horizon	OFF (4 switches in line including Autopilot)

Magnetos			CHECK (2)

Mixture			CUTOFF & wait for engine to stop

Magnetos			OFF

Alternator	field		OFF

Main switch		OFF

Control lock		INSTALL

Pitot and static covers	INSTALL



EMERGENCY PROCEDURES

See Section 3 in the POH for the official emergency procedures;

the Supplements section deals with the Autopilot etc.

Emergency gear extension

Airspeed			97KT OR LESS

Landing gear lever		DOWN

LDG GEAR cct breaker	CHECK PUSHED

Landing gear lights		TEST

Flaps			TAKEOFF

If gear does not extend (cannot see all 3 greens):

Landing gear lever		UP

LDG GEAR cct breaker	PULL

Airspeed			97KT OR LESS

Landing gear lever		DOWN

Emergency gear		PULL

Autopilot Failure

Control wheel		GRASP FIRMLY AND REGAIN CONTROL

RED button		PRESS AND HOLD

Aircraft			RETRIM MANUALLY AS REQUIRED

AP Master Switch		OFF

AP Servos circuit breaker	PULL

Never switch the autopilot back on during flight after the above.



Emergency Comms and Nav

In the event of total electrical failure, switch OFF the COM2/NAV2 radio (because you are about

to disconnect its antenna) and connect the ICOM A22 transceiver and the SKYMAP2 GPS as follows:

The GPS battery life is about 4 hours.

The ICOM battery life is several hours listening but much less when talking.

Interception Procedures